Project:CS3520

M1:

DOMAIN: A SMALL CALCULATOR

CLASS: RISC

Register:32 registers

Word Length: 32 bit

| Register | Sesotho Equivalent | Conversional Use | Width(in bits) |
| --- | --- | --- | --- |
| p0 | Noto | Hard wired to zero | 5 |
| p1 | Hae | Return Address - Set automatically by jal | 5 |
| p2 | Lesupa | Stack pointer | 5 |
| p5-p7, p28-p31 | Nakoana(n0-n6) | Temporaries - A called procedure may destroy these | 5 |
| p8-p9, p18-p27 | Bolokuoe(b0-b11) | Saved - A called procedure may preserve them | 5 |
| p10-p17 | Taba le pusetso(t0-t7) | Arguments and return values | 5 |
| n/a | Sebali | Program Counter | 5 |

| Category | Instruction Format | Snippet | Description | RISC-V Equivalent |
| --- | --- | --- | --- | --- |
| Arithmetic | R-Type | kopanya p5, p5, p6 | Add two registers | add |
| Arithmetic | R-Type | tlosa p6, p7, p5 | Subtract two registers | sub |
| Arithmetic | I-Type | kopanyah p5, p6, 3 | Add intermediate value | addi |
| Logic | R-Type | le p5,p6,p7 | Bitwise and | and |
| Logic | R-Type | kapa p5,p6,p7 | Bitwise or | or |
| Logic | I-Type | chechatsehali (chetse) | Shift to the left | slli |
| Load | I-Type | thsela | Load from memory to a register | lw |
| Save | S-Type | boloka | Save to memory from a register | sw |
| Branch | B-Type | lekana | Branch if equal | beq |
| Branch | B-Type | haelekane | Branch if not equal | bne |
| Category | Instruction Format | Operation | Description | RISC-V Equivalent |
| Arithmetic | R-Type | rD = rs1 + rs2 | Add two registers | add |
| Arithmetic | R-Type | rD = rs1 – rs2 | Subtract two registers | sub |
| Arithmetic | I-Type | rD = rs1 + imm | Add intermediate value | addi |
| Logic | R-Type | rD = rs1 & rs2 | Bitwise and | and |
| Logic | R-Type | rD = rs1 ¦ rs2 | Bitwise or | or |
| Logic | I-Type | rD = rs << imm | Shift to the left | slli |
| Load | I-Type | Rd = MEM[Rs + OFFSET] | Load from memory to a register | lw |
| Save | S-Type | MEM[rS + OFFSET] = rD | Save to memory from a register | sw |
| Branch | B-Type | If rS1 == rS2 jump | Branch if equal | beq |
| Branch | B-Type | If rS1 != rS12 jump | Branch if not equal | bne |
| Jump | J-Type | jal rd, label | Jump and link | jal |

| Mnemonic | Meaning | Operation | RISC-V32I Equivalent |
| --- | --- | --- | --- |
| kopanya | To add | rd = rs1 + rs2 | add |
| tlosa | To subtract | rd = rs1 – rs2 | sub |
| kopanyah | To add an immediate | rd = rs1 + imm | addi |
| tshela | Load to a register | rd = MEM[rs +OFFSET] | lw |
| boloka | To store | MEM[rs + offset] = rd | sw |
| lekana | Branch if equal | If rs1 == rs2 jump | beq |
| haelekane | Branch if not equal | If rs1 != rs2 | bne |
| chechatsehali (chetse) | Shift left logic immediate | rD = rs1<< shamt | slli |
| kapa | Or | rD = rs1¦rs2 | or |
| le | And | rD = rs1 & rs2 | and |
| qhoma_kopanya | Jump and link | qhok rd, label | jal |

A5

| Field | Width | Meaning |
| --- | --- | --- |
| opcode | 7 | Primary opcode |
| rd | 5 | Destination register |
| funct3 | 3 | Operation selector within an opcode |
| rs1, rs2 | 5+5 | Source registers |
| funct7 | 7 | Extra selector (R-type; also the fixed upper bits of chetse) |
| Imm | 12(I,S), 13(B, bit 0 implicit), 21(J, bit 0 implicit) | Immediate, store in scattered layouts below |

R-type - kopanya, tlosa, le, kopa

| 31-25 | 24-20 | 19-15 | 14-12 | 11-7 | 6-0 |
| --- | --- | --- | --- | --- | --- |
| funct7 | rs2 | rs1 | funct3 | rd | opcode |

I-type kopanyah, chetse, tshela, khutla

| 31-25 | 24-20 | 19-15 | 14-12 | 11-7 | 6-0 |
| --- | --- | --- | --- | --- | --- |
| imm[11:0] | rs2 | rs1 | funct3 | rd | opcode |

S-type - boloka

| 31-25 | 24-20 | 19-15 | 14-12 | 11-7 | 6-0 |
| --- | --- | --- | --- | --- | --- |
| imm[11:5] | rs2 | rs1 | funct3 | imm[4:0] | opcode |

B-type lekana, hae lekane, nyane

| 31-25 | 24-20 | 19-15 | 14-12 | 11-7 | 6-0 |
| --- | --- | --- | --- | --- | --- |
| imm[12] | rs2 | rs1 | funct3 | imm[4:1] | opcode |

J-type qhoma_kopanya(qhok)

| 31-25 | 24-20 | 19-15 | 14-12 | 11-7 | 6-0 |
| --- | --- | --- | --- | --- | --- |
| imm[20] | imm[10:1] | rs1 | funct3 | rd | opcode |

| Mnemonic | RV32I | Format | Opcode | Funct3 | Funct7 |
| --- | --- | --- | --- | --- | --- |
| kopanya | add | R | 0110011 | 000 | 0000000 |
| tlosa | sub | R | 0110011 | 000 | 0100000 |
| le | and | R | 0110011 | 111 | 0000000 |
| Kapa | or | R | 0110011 | 110 | n/a |
| kopanyah | addi | I | 0010011 | 000 | 0000000 |
| chetse | sllii | I(load) | 0010011 | 001 | n/a |
| tshela | lw | I | 0010011 | 010 | n/a |
| boloka | sw | S | 0100011 | 010 | n/a |
| lekana | beq | B | 1100011 | 000 | n/a |
| haelekane | bne | B | 1100011 | 001 | n/a |
| qhoma_kopanya | jal | J | 1101111 | n/a | n/a |

Immediate Ranges

| Instruction | Immediate | Range |
| --- | --- | --- |
| kopanyah, tlolela | 12-bit signed | -2048 to 2047 |
| tshela, boloka(offset) | 12-bit signed |  |
| chetse | 5-bit unsigned shift amount | 0 t0 31 |
| Lekana, haelekane | 13-bit signed, even(bit 0 implicit) | -4096 to +4096 bytes |
| tlola | 21-bit signed, even(bit 0 implicit) | -1048576 to +1048574 bytes |

Hand-encoded exampled

| Format | Instruction | Fields | Binary | Hex |
| --- | --- | --- | --- | --- |
| R | kopanya p3, p1, p2 | funct7 = 0000000, rs2 = 00010, rs1 = 00001, funct3 = 000, rd = 00011, opcode = 0110011 | 0000000 00010 00001 000 00011 0110011 | 0x002081B3 |
| I | kopanyah p3, p1, 10 | imm= 000000001010, rs1= 00001, funct3 = 000, rd = 00011, opcode = 0010011 | 000000001010 00001 000 00011 0010011 | 0x00A08193 |
| I | chetse p3, p1, 3 | funct7 = 0000000, shamt =00011, rs1 = 00001, funct3 = 001, rd= 00011, opcode = 0010011 | 0000000 00001 000 00011 0010011 | 0x00309193 |
| S | boloka p2, 4(p1) | imm[11:5] = 0000000, rs2 = 00010, rs1 = 00001, funct3 = 010, imm[4:0] = 00100, opcode = 0100011 | 0000000 00010 00001 010 00100 | 0x0020A223 |
| B | lekana p1, p2, 8 | imm[12] = 0, imm[10:5] = 000000, rs2 = 00010, rs1 = 00001, funct3 = 000, imm[4:1] = 0100, imm[11] = 0, opcode = 1100011 | 0 000000 00010 00001 000 0100 0 1100011 | 0x00208463 |
| J | qhoma_kopanya | imm[20] = 0, imm[10:1] = 0000000100, imm[19:12] = 00000000, rd = 00001, opcode =1101111 | 0 0000000100 00000000 00001 1101111 | 0x008000EF |

Rationale for every entry

| Entry | Rationale |
| --- | --- |
| 32-bit fixed instructions | One fetch per cycle and no length decoding in IF |
| 32-bit registers, 5-bit fields | RV32I-compatible field positions, enough registers that the sample programs never spill |